Week 1: Concept & Research

- Feb 1: Identified the problem of Cosine Loss. Researching how active tracking can keep a panel at a 90° angle.

- Feb 2: Unpacked components. Tested LDR resistance ranges by connecting the LDRs to the Arduino board

- Feb 3: Researching "Differential Logic." The plan is to use the difference between two sensors to drive a servo motor.

- Feb 4: Sketching the first frame design. Decided on a compact, single-axis build.

- Feb 5: The First Prototype Build: Attempted to solder LDRs and resistors directly to the Arduino pins for a "permanent" look.

- Feb 7: CRITICAL FAILURE: The soldering was poor. Mechanical vibrations from the motor caused "cold joints" to crack. I also accidentally bridged two pins with solder, causing a short circuit. Prototype #1 is dead.

Week 2: Rebuilding & Logic Fixes

- Feb 9: The Pivot: Switched to a Breadboard-based design for Prototype #2. The connections are now secure and easy to troubleshoot.

- Feb 10: Rewired the voltage divider on the breadboard. Readings are now stable in the Serial Monitor.

- Feb 11: Problem: The motor is "jittering" (rapid shaking) because the sensors are too sensitive to tiny light flickers.

- Feb 12: The Fix: Implemented a Deadband (threshold of 15 units) in the code. The motor now only moves when the light change is significant.

- Feb 14: Problem: The solar panel is too heavy, causing the motor to "buzz" and stall.

- Feb 15: The Fix: Rebuilt the frame with a Pivot-Axis. Balanced the panel on a dowel so the motor only provides rotation rather than holding the weight.

Week 3: Electrical & Sensor Tuning

- Feb 17: Added a Vertical Blinder between the LDRs to create a sharper shadow. This makes the tracker's response much more accurate.

- Feb 18: Problem: The Arduino reset itself (Brownout) when the motor moved.

- Feb 19: The Fix: Added an external battery pack for the motor and linked the Common Ground (GND) to the Arduino. No more resets.

- Feb 20: Soldered extensions to the LDR legs because the rotation was pulling the wires out of the breadboard.

Week 4: The Testing Week (The Proof)

- Feb 23: Set up the test area with a protractor and a 100W lamp. Measured the "Static" panel baseline output.

- Feb 24: The Breakthrough: Ran Project Helios through a 180° arc. It maintained a 4.8V average, while the static panel dropped to 2.1V at steep angles.

- Feb 25: Tested "Cloudy Conditions" using paper to dim the light. The Deadband logic kept the system stable and prevented "hunting."

- Feb 26: Data Entry Day. Recorded voltages for 10 different angles and calculated a 32.7% total efficiency gain.

- Feb 27: Final Stress Test. Ran the tracker for 4 hours straight to ensure the servo didn't overheat. Everything is stable.